Smart Ambulance

Priority-Based Patient Telemetry on QNX Neutrino RTOS

Detailed Project Documentation

State Level Hackathon

Team: Dharani_aus [Surampalem]

Domain: Healthcare / Emergency Medical Services

Critical alerts never wait behind routine telemetry.

Table of Contents

1. Introduction	3

2. Problem Statement	3

3. Solution Overview	3

4. System Architecture	4

5. Triage Rules	4

6. QNX Concepts Used	5

7. Message Format and Reply Codes	5

8. Code Walkthrough	6

9. Build and Run Instructions	6

10. Test Scenario and Output Walkthrough	8

11. Resilience and Error Handling	11

12. Performance	11

13. Limitations and Known Behaviour	11

14. Future Scope	12

15. Key Learnings	12

# 1. Introduction

Smart Ambulance is a real-time telemetry system that streams patient vitals from an ambulance to a hospital. It is built on the QNX Neutrino RTOS using native microkernel message passing. The core guarantee is simple: a life-threatening reading is always processed before routine data, no matter how much routine traffic is already in flight.

The project consists of two programs:

| Program | Role |
| --- | --- |
| Ambulance Transmitter | Takes patient vitals, validates them, classifies each reading as CRITICAL or ROUTINE, and sends it to the hospital. |
| Hospital Receiver | Receives readings on two separate channels, processes critical alerts on a high-priority thread and routine data on a low-priority thread, and acknowledges every message. |

# 2. Problem Statement

## 2.1 Scenario

An ambulance continuously streams heart rate, SpO2, temperature, GPS position and emergency severity. If the hospital system is busy processing routine updates when a critical reading arrives, the alert is delayed and the ER loses valuable preparation time.

## 2.2 Why it matters

A late alert means late treatment. The critical alert must be handled first, every time.

## 2.3 Success metrics

| Metric | Target |
| --- | --- |
| Critical alerts served before routine work | 100% |
| Time for a running routine job to yield after a critical alert | Within 100 ms |
| Interrupted routine job | Acknowledged to the sender as INTERRUPTED |
| Triage | Applied by the transmitter |

# 3. Solution Overview

The design rests on three ideas:

- Triage at the source. The transmitter decides, per reading, whether it is critical or routine.

- Separate channels and threads. Critical and routine traffic use different QNX channels, each served by its own thread, so critical traffic can never queue behind routine traffic.

- Priority-based preemption. The critical thread runs at SCHED_RR priority 30 and the routine thread at priority 10. A shared, mutex-protected counter lets the long-running routine job notice a critical arrival and yield.

# 4. System Architecture

| Ambulance Transmitter | Hospital Receiver |
| --- | --- |
| Reads vitals; validates ranges; classifies each reading as CRITICAL or ROUTINE | Critical channel > Critical thread (SCHED_RR 30): prints alert at once, increments critical_count, replies CRITICAL |
| MsgSend (critical) ------> | Routine channel > Routine thread (SCHED_RR 10): 5 s job (50 x 100 ms), aborts if critical_count changes, replies INTERRUPTED |
| MsgSend (routine) ------> | Shared critical_count protected by a mutex |
| <------ MsgReply (acts as the ACK) | Reply codes: 1 CRITICAL, 2 ROUTINE, 3 INTERRUPTED, -1 REJECTED |

## 4.1 Design decisions

| Decision | Reason |
| --- | --- |
| Two processes (transmitter, receiver) | Mirrors the real ambulance-to-hospital separation and uses QNX IPC between processes. |
| Native MsgSend / MsgReceive / MsgReply | Synchronous and priority-aware; the reply doubles as the ACK. |
| One channel per priority class | Critical traffic never waits behind routine traffic. |
| _NTO_CHF_FIXED_PRIORITY on both channels | The receiving thread keeps its own fixed priority instead of inheriting the sender priority. |
| SCHED_RR priorities 30 and 10 | Deterministic priority ordering of the two receiver threads. |
| Mutex-protected critical_count | Safe signalling from the critical thread to the routine thread. |

# 5. Triage Rules

Triage is applied by the transmitter before sending. Critical readings go to the critical channel; everything else goes to the routine channel.

| Parameter | CRITICAL if | ROUTINE |
| --- | --- | --- |
| Severity (1-5) | 4 or 5 | 1 to 3 |
| Heartbeat | < 50 or > 130 BPM | 50 to 130 BPM |
| SpO2 | < 90 % | 90 % or more |
| Temperature | > 39 C or < 35 C | 35 to 39 C (inclusive) |
| GPS | Never | Always |

## 5.1 Input validation

Before triage, every reading is range-checked. A set containing any out-of-range value is discarded and the user is asked to re-enter it.

| Field | Accepted range |
| --- | --- |
| Heartbeat | 0 - 300 BPM |
| SpO2 | 0 - 100 % |
| Temperature | 20 - 45 C |
| Latitude | -90 to 90 |
| Longitude | -180 to 180 |
| Severity | 1 - 5 |

# 6. QNX Concepts Used

| Concept | Where | Purpose |
| --- | --- | --- |
| ChannelCreate(_NTO_CHF_FIXED_PRIORITY) | Receiver | Creates the critical and routine channels with fixed receiver priority. |
| ConnectAttach(..., _NTO_SIDE_CHANNEL, ...) | Transmitter | Opens a connection to each channel using the hospital PID and channel IDs. |
| MsgSend | Transmitter | Sends a reading and blocks until the reply (ACK) arrives. |
| MsgReceive | Receiver | Blocks until a message arrives on the thread channel. |
| MsgReply | Receiver | Unblocks the sender and returns the reply code. |
| pthread_setschedparam (SCHED_RR) | Receiver | Sets thread priorities 30 and 10. |
| pthread_mutex_t | Receiver | Protects critical_count. |
| nanosleep | Receiver | 100 ms steps of the simulated routine job. |

# 7. Message Format and Reply Codes

## 7.1 PatientMessage structure

typedef struct {

int    ambulance_id;

int    parameter;   // HR, SPO2, TEMPERATURE, GPS, EMERGENCY

int    priority;    // CRITICAL or ROUTINE

int    severity;    // 1-5

double value;

double latitude;

double longitude;

} PatientMessage;

| Reply code | Value | Transmitter prints |
| --- | --- | --- |
| CRITICAL | 1 | ACK: Critical alert processed |
| ROUTINE | 2 | ACK: Routine data processed |
| INTERRUPTED | 3 | ACK: Routine processing interrupted |
| Rejected | -1 | ACK: Invalid or rejected message |

# 8. Code Walkthrough

## 8.1 Ambulance Transmitter

- Setup: disables stdout buffering, then asks for the Ambulance ID, Hospital PID, Critical Channel ID and Routine Channel ID (printed by the receiver at start-up).

- Connect: calls ConnectAttach twice to obtain critical_coid and routine_coid.

- Main loop: for each patient data set it reads HR, SpO2, temperature, latitude, longitude and severity, then range-checks them.

- Critical first: severity 4 or 5 sends EMERGENCY; out-of-range HR, SpO2 or temperature each send a CRITICAL message.

- Routine next: in-range HR, SpO2 and temperature, then GPS, then EMERGENCY if severity is below 4.

- send_parameter(): builds the message, picks the connection by priority, calls MsgSend, and prints the ACK based on the reply code.

## 8.2 Hospital Receiver

- Creates two channels with _NTO_CHF_FIXED_PRIORITY and prints the PID and both channel IDs.

- Starts two threads: critical_receiver (priority 30) and routine_receiver (priority 10).

- Critical thread: rejects non-critical messages with -1; otherwise increments critical_count under the mutex, prints the alert, and replies CRITICAL.

- Routine thread: rejects non-routine messages with -1; otherwise records critical_count and runs a 5-second job (50 x 100 ms). After each step it checks whether critical_count changed. If so, the job aborts and replies INTERRUPTED; otherwise it replies ROUTINE.

# 9. Build and Run Instructions

## 9.1 Requirements

- QNX Neutrino RTOS (QNX SDP) target or VM

- QNX Momentics IDE or the qcc toolchain

- Privileges to set SCHED_RR priorities

## 9.2 Build

qcc -o hospital_receiver     hospital_receiver.c

qcc -o ambulance_transmitter ambulance_transmitter.c

## 9.3 Run

- Start the receiver and note the printed Hospital PID, Critical Channel ID and Routine Channel ID.

- Start two transmitters in separate consoles, each with a different Ambulance ID, entering the PID and channel IDs from step 1.

- Enter patient readings in each transmitter.

Note: two transmitters are needed to demonstrate preemption. A single transmitter blocks on MsgSend until each reply returns, so it cannot send a critical alert while its own routine message is still being processed.

# 10. Test Scenario and Output Walkthrough

The test was run in the QNX Momentics console with three consoles side by side: the Hospital Receiver (left) and two Ambulance Transmitters (middle: Ambulance 1, right: Ambulance 2).

Hospital PID: 532501     Critical channel: 1     Routine channel: 2

## 10.1 Inputs

|  | Ambulance 1 | Ambulance 2 |
| --- | --- | --- |
| Heartbeat | 96 BPM | 95 BPM |
| SpO2 | 95 % | 82 % |
| Temperature | 39 C | 36 C |
| Latitude / Longitude | 17 / 81 | 17 / 81 |
| Severity | 2 | 4 |
| Expected triage | All ROUTINE (39 C is the upper limit of the routine range) | EMERGENCY (severity 4) and SpO2 (82 < 90) are CRITICAL; the rest are routine |

## 10.2 Step 1: Critical alerts arrive during routine processing

Figure 1: Critical alerts from Ambulance 2 preempt routine processing of Ambulance 1

- The receiver prints both thread priorities being activated: SCHED_RR priority 30 and 10.

- Ambulance 1 routine HEARTBEAT (96 BPM) starts processing (ROUTINE PROCESSING STARTED).

- While that job is running, Ambulance 2 critical messages arrive: first EMERGENCY SEVERITY 4/5, then SpO2 82 %. Each is printed immediately with "CRITICAL ALERT RECEIVED" and "Processed by HIGH PRIORITY thread".

- The receiver then prints "ROUTINE INTERRUPTED! Critical alert took priority."

- Ambulance 1 console shows "ACK: Routine processing interrupted". Ambulance 2 console shows "ACK: Critical alert processed" for both critical messages.

## 10.3 Step 2: Routine processing resumes

Figure 2: Interruption message, then routine processing resumes with the next messages

- The receiver moves on to the next routine messages: Ambulance 2 HEARTBEAT 95 BPM, Ambulance 1 SpO2 95 %, and Ambulance 2 TEMPERATURE 36.0 C.

- Each prints ROUTINE PROCESSING STARTED followed by ROUTINE PROCESSING COMPLETED, since no new critical alert interrupted them.

## 10.4 Step 3: Remaining routine data is processed

Figure 3: Routine messages from both ambulances are processed in turn

- Ambulance 1 TEMPERATURE 39.0 C, Ambulance 2 GPS (17.000000, 81.000000), Ambulance 1 GPS and Ambulance 1 EMERGENCY SEVERITY (routine) are processed in turn, interleaved between the two ambulances.

## 10.5 Step 4: All parameters submitted

Figure 4: Final routine message completes and both transmitters are ready for new readings

- The last routine message (Ambulance 1, Emergency severity 2/5) completes with ROUTINE PROCESSING COMPLETED.

- Both transmitters print "All patient parameters submitted." followed by "Enter next set of readings."

## 10.6 What the output proves

| Observation | What it shows |
| --- | --- |
| Critical alerts print immediately while a routine job is running | The high-priority thread preempts routine work. |
| ROUTINE INTERRUPTED followed by ACK: Routine processing interrupted | The routine job detects the critical arrival and yields; the sender is informed. |
| Ambulance 2 critical messages served before remaining routine messages | Order is decided by priority, not arrival time. |
| Both ambulances served interleaved | The receiver handles multiple ambulances at once. |
| Undisturbed routine messages complete normally | Normal operation is unaffected. |

# 11. Resilience and Error Handling

| Risk | Mitigation |
| --- | --- |
| Critical alert arrives during a 5 s routine job | Dedicated high-priority thread and channel; the routine job polls every 100 ms and aborts, replying INTERRUPTED. |
| Several ambulances transmit at once | Each message carries an ambulance_id; the critical channel is served by the high-priority thread regardless of sender. |
| Invalid readings | Transmitter range-checks every reading and discards bad data sets. |
| Message sent on the wrong channel | Receiver replies -1; transmitter prints "Invalid or rejected message". |
| MsgReceive / MsgSend failure | Errors are reported with perror; the receiver loops and the transmitter returns from the send. |
| Insufficient scheduling privileges | Receiver prints a clear message and exits if pthread_setschedparam fails. |
| Concurrent access to critical_count | Protected by a mutex. |

# 12. Performance

| Metric | Value |
| --- | --- |
| Critical thread priority (SCHED_RR) | 30 |
| Routine thread priority (SCHED_RR) | 10 |
| Routine job length | 50 x 100 ms = 5 s |
| Maximum time for routine work to yield (design) | 100 ms (one polling step) |
| Measured critical alert latency | To be filled from the QNX Momentics System Profiler |

# 13. Limitations and Known Behaviour

- Interrupted routine messages are not retried. The receiver acknowledges them as INTERRUPTED and moves on. In the test above, the Ambulance 1 HEARTBEAT reading was dropped this way; the transmitter is informed through the ACK but does not resend it.

- Console input only. No GPIO or real sensors in this build.

- Polling, not events. Interruption is detected by checking a counter every 100 ms.

- Simulated routine work. The 5-second routine job is a simulated delay.

- Single routine thread. Routine messages from all ambulances are processed one at a time.

- No CPU sets or affinity, and a plain mutex rather than a priority-inheritance mutex.

# 14. Future Scope

| Area | Idea |
| --- | --- |
| Real sensors | Pulse-oximeter, temperature sensor and GPS over UART or CAN replace console input. |
| Network scale | Send the same messages over QNX Qnet to a remote hospital node, with many ambulances. |
| Event-driven design | Replace 100 ms polling with an event or pulse for faster interruption. |
| Safety hardening | Priority-inheritance mutex, HAM auto-restart, CPU affinity, logging. |
| Reliability | Retry or re-queue interrupted routine messages. |

# 15. Key Learnings

- A blocking MsgReceive server cannot be interrupted mid-job.

- The fix is priority-separated channels and threads.

- A mutex-protected counter, polled every 100 ms, lets the low-priority job yield to critical work.

- Priority, not arrival order, should decide what gets processed first in safety-critical systems.

Every second counts: the right alert reaches the ER first, every time.